| When work activity or hazard changes significantly or an incident occurs, review this SWMS or prepare another & consult with team on revised / changed issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BUSINESS DETAILS of the BUSINESS PREPARING THIS SAFE WORK METHOD STATEMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Name | Kwikserv Formwork & Scaffolding (VIC) Pty Ltd |  | Contact Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BN / ACN | 67 110 883 817 |  | Contact Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address | 21 -23 Metcalf Drive, Dandenong South |  | Contact Phone Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT & WORK DETAILS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Client Name |  |  | Work Manager to assess special requirements and additional measures if required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project Name |  |  | Engineering Requirement |  |  |  |  | Yes |  | No |  | Restricted access |  |  |  |  | Yes |  | No |
| Project Location |  |  | Permit to Work System |  |  |  |  | Yes |  | No |  | Other workers |  |  |  |  | Yes |  | No |
|  |  |  | Special training, specify below |  |  |  |  | Yes |  | No |  | Special supervision |  |  |  |  | Yes |  | No |
| Work Description | Scaffold Erection |  | Work Duration |  |  | 5min |  |  | Hour |  | ½ day |  | Day |  | Week | Month |  | Year |  |
|  | Scaffold erection, alteration & dismantling |  | Work Manager |  |  |  |  |  |  |  | Position |  |  |  |  |  |  |  |  |
|  |  |  | Is the person responsible for implementing and monitoring control measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personnel involved | Own workers / Hired in workers |  | Plant Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| & licenses or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| competencies req’d | General Induction Card, Red Card or equivalent |  | Plant Mobile |  | Scaffold items |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High Risk Work | Scaffold over 4m, HRW Licence required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Working at Height |  | Equipment Used |  | Hammer and ratchet mainly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Material & | Kwikstage modular scaffold |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Substances used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Maintenance |  | Daily scaffold check before commencing work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Checks & |  | 4 weekly scaffold check after hand over |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Frequency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Subcontractors Used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List by Org Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| PERSONAL PROTECTIVE EQUIPMENT applying to general WORK ACTIVITES – add additional for specific work tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LEGISLATION, CODES & STANDARDS applying to general WORK ACTIVITES – add additional for specific work tasks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Legislation | Codes & industry Standards |  |  |  |  |  |  |  |  |  |  |  | Standards |  |  | List others below |  |
| OHS Act 2004 | CC Prevention of Falls in General Construction |  |  |  |  |  |  |  |  |  |  |  | AS/NZ 1801 Hard Hats |  |  | AS 1576.1 2010 & AS4576 Scaffolding |  |
| OHS Regulations 2007 | COP Manual Handling, COP Plant, |  |  |  |  |  |  |  |  |  |  |  | AS/NZ 2210 Protective Footwear |  |  |  |  |
| EPA Act 1970 |  |  |  |  |  |  |  |  |  |  |  |  | AS/NZ 4602 High Visibility Safety Garment |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | AS/NZ 1337 & 1270 Eye & Ear Protection |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | AS/NZ 3102 & 3760 Testing & Tagging |  |  |  |  |
| WORK LOCATION CONDITIONS & CONTROLLING HAZARDS – complete for work location & task conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work location conditions – working safely may be affected by site conditions |  |  |  |  |  |  |  |  |  |  | When considering how to control hazards, use Hierarchy of Control below |  |  |  |  |  |  |
| Access - site & work area |  | Yes | No | Overhead services |  | Yes | No |  |  |  | 1. Elimination – eliminate the work practice, plant and / or equipment responsible |  |  |  |  |  |  |
| Contaminants, fumes |  | Yes | No | Public Interruption |  | Yes | No |  |  |  | 2. Substitution – substitute the work practice, plant and / or equipment for a safer alternative |  |  |  |  |  |  |
| Ground uneven, soft |  | Yes | No | Traffic on-site |  | Yes | No |  |  |  | 3. Isolation – isolate the work practice, plant and / or equipment responsible |  |  |  |  |  |  |
| Noise, dust, vibration |  | Yes | No | Underground services |  | Yes | No |  |  |  | 4. Engineering – redesign work practice, material use, plant or equipment for a safer alternative |  |  |  |  |  |  |
| Objects, obstacles |  | Yes | No | Visibility, line of sight |  | Yes | No |  |  |  | 5. Administrative – change the procedures to reduce exposure (job rotation, training, etc) |  |  |  |  |  |  |
| Other works on site |  | Yes | No | Wind, rain, cold, heat |  | Yes | No |  |  |  | 6. Personnel Protective Equipment - use suitable equipment / apparel to reduce risk / exposure |  |  |  |  |  |  |
| Other site conditions affecting safe work? List below |  |  |  |  |  |  |  |  |  |  | Always think and act from top down, ie firstly eliminate hazard. If not reasonably |  |  |  |  |  |  |
|  |  | Yes | No |  |  | Yes | No |  |  |  | practical to do so, then move to lower level control |  |  |  |  |  |  |
|  |  | Yes | No |  |  | Yes | No |  |  |  |  |  |  |  |  |  |  |
| RISK ASSESSMENT PROCESS – to determine Risk Level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| To determine Risk Level of potential hazard, use the risk matrix and follow the process below. |  |  |  |  |  |  |  |  |  |  |  | Risk Level Matrix Occurrence Consequence; Occurrence Probability. Frequent; Probable; Occasional; Remoteeeeeee. Catastrophic; High; High; High; High. Fatality; High; High; High; Medium. Permanent Disability; High; High; Medium; Low. Lost Time Injury; High; Medium; Low; Low. Medical Treatment; Medium; Low; Low; Low. First Aid Injury; Low; Low; Low; Low. |
| 1. First, consider the probability of the potential hazard actually causing injury, illness |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or incident. Don’t over complicate your thoughts. What is the probability of this |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| hazard causing injury – Frequent, Probable, Occasional or Remote? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Then consider the consequence of the injury, illness or incident caused. Keep it |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| simple, don’t dream up a catastrophic event when the realistic likelihood is medical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| treatment – Catastrophic, Fatal, Permanent Disability, Lost Time Injury, Medical |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| treatment, First Aid Injury. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Risk Ranking The resultant position on the Risk Matrix provides a risk ranking of |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High, Medium or Low. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Now control hazards. First deal with hazards having a High rank, then |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| edium, then Low. Consider ‘Hierarchy of Control’ above when determining suitable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| control measures, ie use highest control where reasonably practicable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Assess ‘Residual Risk’ level after control applied. What else needs doing? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| CONSULTATION DETAILS and ON-SITE ACKNOWLEDGEMENT – complete for this work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| This Safe Work Method Statement has been developed in consultation with our workers, including any subcontract workers involved in the work, & the H&S Rep. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prepared by | Kwikserv Formwork and Scaffold |  |  | Date |  |  |  |  |  |  |  |  |  |  | Notes & |  |  |  |  |  |  |  |
| Reviewed & Approved by |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Agreements |  |  |  |  |  |  |  |
| Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approver Signed |  |  |  | Date |  |  |  |  |  |  |  |  |  |  | H&S Rep Name |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | H&S Rep Signed |  |  |  |  |  |  |  |
| Next Review by |  |  |  | Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date |  |  |
| I was consulted, took part in the preparation, have read, understood and been trained in the requirements of this Safe Work Method Statement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NAME |  |  | SIGNED |  |  | DATE |  |  |  |  |  |  |  | NAME |  |  |  |  | SIGNED |  | DATE |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
|  |  |  |  |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  | / / |  |
| PRINCIPAL CONTRACTOR APPROVAL – have principal contractor approve SWMS for work |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Principal Contractor (PC) |  |  |  |  |  |  |  |  |  |  |  |  | Notes & Agreements |  |  |  |  |  |  |  |  |  |
| PC Representative Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PC Representative Sign off |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Date |  |  | / / |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| WORK ACTIVITY Step by step work to perform activity in sequence work carried out | Potential Hazard Potential hazards may cause incident / injury | Risk Level | Control Measures Control measures required to eliminate or minimise risk of incident / injury | Residual Risk Level | Person to implement this control |  |
| --- | --- | --- | --- | --- | --- | --- |
| Preparation |  |  |  |  |  |  |
| Induct & check competencies Determine mechanical aids required, arrange delivery / procurement / issue from store Examine the area over and around the work area in particular, be aware of overhead powerlines, traffic and other persons on site. Uneven grounds which result in slip and fall injuries, hazards such as rubbish and debris around the site can lead to risk or trip injuries Underground Services, check location of underground services, Water, Gas & Phone | Personal injury from inadequate competencies or interaction with substances The weight the scaffold erected on top of Underground Services may damage them | H H H H H | Induct workers into work requirements, check worker competency & experience, train as required, supervise worker if inexperienced, check work capability for plant, equipment & tools (EWP’s, laser, etc) including operating instructions & review material safety data sheets (MSDS) Ensure all persons involved in the erection, alteration & dismantling of scaffolding have the appropriate training & licenses for work undertaken Prior to work commencement, ensure that you carry out a site-specific assessment, examine for slip and fall hazards, clean up any rubbish and debris around the work site before job commencement If power lines are within 4M to 6M laterally and 5 meters vertical, principal contractor to provide relevant permits from power supplier for all scaffold structures erected within 4M to 6 M laterally, 5M vertical of live power feeds, leading hand scaffolder to ensure that all permits in place Identify locations of all underground services, avoid erecting scaffold on top of underground services, ensure adequate load distribution | M M M M M | Supervisor & Workers |  |
| Discuss work process with workers – tool box meeting | Incorrect Equipment use causing injury | M | Ensure worker familiar with equipment & tool operation. Briefing to all members of the team on work to be per-formed allocation of duties based on skills, experience and abilities All workers must on arrival to any new work site report to site office and receive site induction prior to commencing work | L | Supervisor & workers |  |
|  | Personal injury arising from work process | M | Wear protective clothing & equipment, hi-vis clothing footwear, eye & ear protection, gloves as required | L | Supervisor & Workers |  |
| Discuss health issues with worker | Person injury incurred from slip, trip sprains, strains | M | Share loads where carrying greater than 20 kg’s, bend knees when lifting, for heavy loads use mechanical aids like crane, wear protective clothing & equipment like gloves, hard hat | L | Supervisor & Workers |  |
|  | Working near live electrical lines and incurring electric shock or electrocution | H | Identify electrical lines in work area above and below ground. Where present, do not commence work until distribution company confirms electricity is isolated or follow ‘No Go Zone’ rules | M | Supervisor & Workers |  |
|  | UV Exposure, dehydration, heat & sun stress | M | Provide cool drinking water, consume liquids, keep hydrated. Provide and use suitable PPE including sunscreen, full shoulder to knee and elbow length clothing, UV rated sunglasses &hard hats with brim | L | Supervisor & Workers |  |
|  | Dust, inhalation | M | Use goggles, gloves, good ventilation, have first aid kit nearby | L | Supervisor & Workers |  |
|  | Working in ‘Permit to Work’ Areas | H | Obtain Permit to Work – Hot Work / Laser Use / Work at Heights, if required and discuss issues | M | Supervisor & Workers |  |
| Discuss emergency response / rescue plan refer FORM Emergency Response Plan | Arising from fall, medical situation, etc | H | Refer Emergency Response Plan | M | Supervisor & Workers |  |
| Operations Plan scaffold works: Determine the point at which to start building the scaffold – placement of first standard | Overhead services, electric shock Incorrectly structured & misaligned scaffolding, need to be dismantled & re-erected | H H | Check overhead services, electric cables, etc. Liaise with Authorities. Comply with IS Electrical Installations at Construction Sites including install RCD (Residual Current Device) on switchboards & or generators Take appropriate precautionary measures to safely isolate the public from work areas, install any necessary barricading, signs, hoarding, lights, etc Ensure all drawings & information is obtained prior to starting | M M | Supervisor & Workers Supervisor & Workers |  |
|  | Overloading components | H | Choose correct type of scaffold. Ensure design limits suitable to loads proposed | M | Supervisor & Workers |  |
|  | Inadequate communication | H | Nominate liaison officer to communicate with principal contractor & sub-contractors. Provide adequate consultation to workers | M | Supervisor & Workers |  |
| Check for obstacles, obstructions, base conditions, underground, overhead services Determine / arrange scaffold & delivery | Working near live electrical lines | H | Identify electrical lines in work area above and below ground. Commence work once Principal Contractor or distribution company confirms electricity is isolated or follow ‘No Go Zone’ rules | M | Supervisor & Workers |  |
| Deliver materials | Vermin, trip, slip | H | Clear walk areas & wear PPE such as protective footwear | M | Supervisor & Workers |  |
|  | Bogging, tipping, damage | H | Designate unload points, stabilize ground, clear obstructions & obstacles | M | Supervisor & Workers |  |
|  | Collision & injury | H | Erect barriers for designated vehicle and plant operation zones & display signage. Use spotter when vehicles or plant reversing | L | Supervisor & Workers |  |
| Unload materials | Roll over | M | Check for pits, sumps, etc, inform driver & others | L | Supervisor & Workers |  |
|  | Drive, kerb damage | M | Unload on road or site, use barriers & spotter as needed | L | Supervisor & Workers |  |
|  | Person injury, sprain, strain | M | Use team lifting & follow manual handling procedures e.g. maintain straight posture, use legs & lower b H M L ack muscles, only moving forward | L | Supervisor & Workers |  |
|  | Injury, heat, UV | M | Ensure persons qualified for task, wear protective gear, gloves, boots, helmets, apply sunscreen | L | Supervisor & Workers |  |
| Transport materials to work / erection areas | Slips, trips, sprains, strains | M | Watch blind corners. Use anti slips strips on ramps. Follow lifting procedure | L | Supervisor & Workers |  |
| Check scaffold components before erecting Inspect all scaffold and identify any faulty equipment | Mixing components | M | Check for compatible components | L | Supervisor & Workers |  |
|  | Damaged components Defects in scaffold components can result in injuries from collapse | M | Remove, don’t use damaged components Clearly mark any faulty components and set aside, notify Kwikserv of defective scaffolding components and arrange to have the equipment returned to Kwikserv for repair | L | Supervisor & Workers |  |
| Sort components for erection – assess the ground conditions | Personal Injury | M | Wear PPE, share loads, bend knees, use mechanical aids eg forklift (check for HRW License) | L | Supervisor & Workers |  |
|  | Stack collapse Uneven or soft ground can result in unsteady and unstable scaffold which could collapse | H | Check correct stacking procedures, stillages stacked correctly. Ensure that the ground is compacted or levelled out prior to job commencement of the installation so that adequate bearing pressure for scaffold is obtained as per Australian standards | M | Supervisor & Workers |  |
| EMERGENCY RESPONSE PROCEDURES – attach specific emergency procedures where required |  |  |  |  |  |  |

Where high risk work is to be undertaken, prepare Emergency Response Plan. Use FORM Emergency Response Plan. Additionally, prior to work being carried out using an Elevated Work Platform (EWP – Boom Lift or Scissor Lift) all effected workers to made familiar with emergency recovery procedures including EWP emergency decent controls. The following procedure may be used when a worker requires rescuing from an EWP for whatsoever reason. - Contact emergency services. Telephone 000. Assign another person to notify Site Supervisor of emergency situation and actions taken. - Assess emergency situation to determine that any rescue worker does not place themselves at risk during any rescue attempt. - After ensuring no person is underneath EWP, a competent person and familiar with EWP ground controls should attempt to carefully lower the EWP platform. - Where EWP lowering is not practical, where suitable, use the following retrieval methods. Crane (tower or mobile) man box. EWP Boom Lift. - Be aware a worker suspended in a harness is at risk of Suspension Trauma. Time is critical. Commence rescue as soon as practical. Encourage suspended person to ‘pump’ their legs to activate leg muscles and reduce potential venous pooling. Where possible, provide method for suspended worker to support self and assist leg ‘pump’ eg use foothold or worker to position body in horizontal or leg high near horizontal position to reduce pressure around upper inside leg circulatory system.